KINETICS

Introduction to reaction kinetics

A chemical reaction proceeds in time. The velocity of a reaction, also called the reaction rate, is defined by the change of the quantity of a chemical component taking part in that reaction. The quantity of a component is proportional to its concentration, so in a constant volume reactor the reaction rate can be calculated from the change of the concentration as well.

The amount of substance is a base quantity in SI, it's unit is a base unit, the usual notation is:

1 mol substance contains NA = 6.023·1023 particles, the mass of the substance is

where M is the molar mass. We denote the volume with V, the concentration is defined by

.

The concentration of a particular chemical component A is:

Let us consider a general chemical reaction. There are several starting materials (Si) and products (Pj), each of them having a stoichiometric number :

We define an abstract quantity , the extent of the reaction, which is independent of the choice of a particular component and of the reaction conditions, eg. the volume may vary, there may be two or more phases, etc. The infinitesimal change of the reaction extent reads:

Now the rate of the reaction is defined as

In the special case of constant volume, we have:

Experimentally, we usually measure concentrations versus time, and obtain concentration rates:

Note that the concentration rates are positive for products and negative for starting materials.

The reaction rate depends mainly from the concentrations and the temperature. The rate of heterogeneous reactions depends strongly from the reaction surface too. For a constant volume, isotherm, and homogeneous reaction the experimentally measured concentration rate can be approximated as

The order of the reaction with respect of the component Si is i and the overall order is i. The experimentally measured rate of a brutto reaction may be the result of many unknown elementary steps and therefore a wide variety of reaction orders can be observed. It is believed that for the elementary steps the so-called mass action kinetics holds, that is the reaction order with respect of a component may be integer one or two or in some special cases three. This is a consequence of a simple collision theory for chemical reactions. An elementary reaction step is monomolecular if one molecule spontaneously changes without any collision of another molecule. In a bimolecular step the reaction needs the collision of two molecules. Trimolecular reactions are rare because of the small possibility of the collision of three molecules. One has always distinguish molecularity and reaction order, however, in the brief  introductions to reaction kinetics only the simple first, second, and third order reactions are mentioned.

The coefficient k is independent from the concentrations and therefore it is called rate constant. Note that the unit of k depends on the overall order of the reaction. The rate constant depends strongly from the temperature T. It is a general rule, that any reaction is faster at higher temperatures. The Arrhenius's law is a good approximation for the temperature dependence of k:

To tell the truth, A is not a constant, it varies slowly with the temperature, so its name is preexponential factor. E is called activation energy and R is the universal gas constant, R = 8.314 J/molK. In a simple model of a reaction, the particles need some excess energy to react, this is the activation energy. Not all collisions result in reaction, only the ones having the activation energy. The exponential factor can be deduced from the Maxwell-Boltzmann distribution. If the activation energy is high, the reaction is slow, and the elevation of the temperature accelerates the rate in a great extent. On the other hand, if the activation energy is low, the reaction is fast but it can be accelerated by the elevation of  the temperature only a little.

Computer simulation of chemical reactions

A general reacting system may consist of many components and may have many reactions. The concentration rate of a selected component is the algebraic sum of  rates originated from different reactions. The general form of the differential equation system reads:

or with matrix notation:

where the components of the vector x are the concentrations. This type of differential equation system can be solved relatively simply by the Euler algorithm:

The Runge-Kutta method replaces Euler's first-order approximation for x with a fourth-order one. However, difficulties arise when there are magnitudinal differences between concentrations of different components or of rate constants of different reactions, these are the so-called stiff systems. For such cases the Rosenbrock algorithm is better.

The system may be strongly non-linear. There is a separate discipline called non-linear dynamics dealing with the behaviour of these systems. Interesting phenomena, for example oscillations, multistability, chaos, etc. may occur.

The set of concentrations obeying the equation

is called stationary point. In simple systems the stationary point represents the end of the reaction. On the other hand, in a more complex system there may be multiple stationary points, furthermore, stationary points may be centres of oscillations too as in the Lotka-Volterra scheme.

Some selected reacting systems

1) First order reaction

Scheme:

Kinetic equations:

Solution:

2) Second order reaction

Scheme:

Kinetic equations:

3) Third order reaction

Scheme:

Kinetic equations:

Remark: after a quick beginning the reaction needs a remarkable time to complete.

4) Equilibrium reaction

Scheme:

Kinetic equations:

Remark: in the equilibrium, the equlibrium constant K is

5) Consecutive reaction

Scheme:

Kinetic equations:

Remark: note that the intermediates go through a maximum, the later ones and the end product appear some time after the beginning .

6) Concurrent reactions

Scheme:

Kinetic equations:

Remark: the product ratio is

7) Catalytic reaction

Scheme:

Kinetic equations:

Remark: note that the reaction rate is proportional to the amount of the catalyst.

8) Autocatalytic reaction

Scheme:

Kinetic equations:

Remark: observe the steep increase of the product concentration after the lag period.

9) Lotka-Volterra oscillating model

Scheme:

Kinetic equations:

10) Michaelis-Menten enzyme kinetics

Scheme:

Kinetic equations:

Remark: In the original terminology, A is the substrate (S), B is the product (P), C is the enzyme (E), and D is the enzyme-substrate complex (ES). The total amount of the enzyme is a constant:

The rate determining step (the slowest from the consecutive ones) is the conversion of the complex, so k3 << k1, k2. Under these circumstances, the concentration of the intermediate D is approximately constant (Bodenstein-principle):

where KM is the Michaelis-constant, nearly equal to the dissociation constant of the complex.

and

While the substrate A is in a great excess, a >> KM and the concentration rate of the product is nearly independent of the substrate concentration: this is an example of a zero order reaction.